# How to Compile a Facility Scope of Work Manually

A step-by-step working guide for turning a facility need into a document contractors can understand, price, perform, and close out.

| Project | Facility | Prepared by | Date |
| --- | --- | --- | --- |

# What this guide is for

This guide is for someone who has never assembled a Scope of Work. It shows the manual process behind the questions asked by Scope-to-Go. Work through the steps in order to see why the platform collects certain information, which decisions still belong to the facility manager, and how the answers become requirements contractors can understand and price.

## How to use this guide

Read the explanation at the start of each step before answering the worksheet prompts.

Use the “Why Scope-to-Go asks” note to connect the manual decision to the platform question.

Do not guess when information is unknown; label it for contractor verification or qualified-specialist review.

At the end, assemble the completed material in the recommended final document order.

# What the finished SOW should accomplish

Give every contractor the same understanding of the required result.

Identify what is included, excluded, owner-furnished, and contractor-furnished.

Describe site conditions and constraints that affect price or execution.

Connect each task to a deliverable or observable completion standard.

Create a common basis for estimates, clarifications, acceptance, and closeout.

# Recommended final document order

| Section | What belongs there |
| --- | --- |
| 1 Purpose and background | Why the work is needed and what condition or business need it addresses. |
| 2 Scope boundaries | Where the work applies, what is included, and what is expressly excluded. |
| 3 Existing conditions | Known facts, records, photos, measurements, asset data, and items requiring verification. |
| 4 Detailed work requirements | The work sequence, materials, responsibilities, protections, testing, and restoration. |
| 5 Schedule and site constraints | Work hours, access, outages, sequencing, coordination, security, and milestones. |
| 6 Deliverables and closeout | Submittals, reports, warranties, manuals, training, photographs, and turnover. |
| 7 Acceptance standards | How completion and quality will be inspected and approved. |
| 8 Pricing instructions | Base price, alternates, allowances, unit prices, exclusions, and assumptions. |

# Step 1  Define the need and desired result

Start with the reason for the project, not a preferred contractor solution. A concise problem statement keeps the scope centered on the required outcome.

Why Scope-to-Go asks about this  Scope-to-Go asks about the problem and desired result because a contractor cannot recommend or price the right work if the platform only knows the requested product or repair.

Describe the present problem or condition in plain language.

Explain why the work is needed now and what operational consequence exists if it is not corrected.

State the finished condition or result the organization needs.

Identify who uses the space or asset and who must approve the outcome.

Write it here  Complete this sentence: The organization needs this work because…

________________________________________________________________________________

________________________________________________________________________________

## Example

Weak: Replace the lobby flooring.
Stronger: Remove the damaged lobby flooring and provide a durable finished floor suitable for daily public traffic, rolling equipment, routine wet cleaning, and continuous access to the reception desk during business hours.

# Step 2  Establish the scope boundaries

Define exactly where the contractor’s responsibility begins and ends. Boundaries prevent contractors from pricing different projects.

Why Scope-to-Go asks about this  The platform asks where the work starts and stops because unspoken boundaries create different assumptions about demolition, patching, cleanup, adjacent finishes, and work by others.

Name the facility, floor, room, system, asset, and limits of the work area.

List included demolition, preparation, installation, patching, painting, restoration, cleanup, and disposal.

List work that is intentionally excluded or will be completed by others.

Identify adjacent spaces, systems, or finishes that must be protected.

| Included work | Excluded or by others | Protection required |
| --- | --- | --- |

# Step 3  Collect and separate project information

Do not mix verified facts, owner decisions, and contractor verification into one undifferentiated list. Label the status of each item so bidders know what they may rely on.

Why Scope-to-Go asks about this  The platform separates confirmed facts, facility-manager decisions, contractor verification, and specialist review so an assumption is not accidentally presented as a requirement or verified condition.

Gather photographs, drawings, measurements, inspection reports, asset records, manuals, prior repairs, and applicable organization standards.

Confirm facts that can be verified internally.

Record decisions the facility manager or owner must make before issue.

Assign inaccessible or field-dependent conditions to contractor verification.

| Information item | Status | Source or responsible party | Why it affects the work |
| --- | --- | --- | --- |
| Example: Existing floor area is approximately 1,250 square feet | Confirmed fact | Facility drawing and field check | Quantity and material pricing |
| Example: Moisture condition below existing floor | Contractor to verify | Pre-bid site investigation | Preparation method and warranty |

## Use precise status labels

Confirmed fact  Supported by an identified record or observation.

Facility manager decision  A choice the owner must make before contractors price the work.

Contractor to verify  A condition bidders must examine or test before submitting a proposal.

Qualified specialist review  A technical, code, life-safety, engineering, or legal matter outside ordinary FM judgment.

# Step 4  Break the work into ordered tasks

Write the work in the order a competent contractor would generally encounter it. Use one accountable action per requirement and identify the result of that action.

Why Scope-to-Go asks about this  The platform asks focused task questions because broad phrases such as “replace the unit” conceal preparation, protection, connections, testing, restoration, and closeout work.

Mobilization, permits, submittals, and pre-work coordination.

Protection, shutdown, isolation, and temporary-service measures.

Removal, demolition, investigation, and preparation.

Installation, repair, configuration, or recurring service tasks.

Testing, correction, restoration, cleanup, and closeout.

| Task number | Contractor action | Required result or evidence |
| --- | --- | --- |
| 1 |  |  |
| 2 |  |  |
| 3 |  |  |
| 4 |  |  |
| 5 |  |  |
| 6 |  |  |

# Step 5  Assign responsibilities

For every supporting activity, identify whether the owner or contractor is responsible. Avoid phrases such as “as necessary” when responsibility matters to price.

Why Scope-to-Go asks about this  The platform asks who is responsible because access, permits, shutdowns, disposal, lifts, and moving furniture all affect price even when they are not the main trade work.

Access, keys, escorts, badging, and security coordination.

Utilities, shutdowns, lockout or isolation, and temporary services.

Permits, inspections, fees, testing, and specialist coordination.

Lifts, dumpsters, storage, staging, parking, and material handling.

Furniture moving, occupant notices, protection, cleanup, and disposal.

| Responsibility | Owner | Contractor | Details or limits |
| --- | --- | --- | --- |
| Site access | ☐ | ☐ |  |
| Permits and inspections | ☐ | ☐ |  |
| Shutdown coordination | ☐ | ☐ |  |
| Furniture or equipment moves | ☐ | ☐ |  |
| Temporary protection | ☐ | ☐ |  |
| Debris removal and disposal | ☐ | ☐ |  |
| Testing and commissioning | ☐ | ☐ |  |

# Step 6  Specify materials and technical requirements

Use performance requirements when the desired result matters more than one method. When a specific product is required, state the characteristics that must be matched and whether substitutions are allowed.

Why Scope-to-Go asks about this  The platform asks about products and standards to distinguish a true requirement from a preference and to make substitution rules understandable to every bidder.

Identify required product type, performance, compatibility, finish, dimensions, and warranty.

Reference applicable drawings, manufacturer instructions, organization standards, codes, or consensus standards.

State whether substitutions are prohibited, permitted as equal, or require written approval.

Require submittals or samples early enough for meaningful review.

| Requirement | Required characteristic or standard | Substitution rule | Submittal required |
| --- | --- | --- | --- |

# Step 7  Document site and schedule constraints

Constraints should be factual, specific, and tied to contractor planning or pricing.

Why Scope-to-Go asks about this  The platform asks about dates, access, occupancy, and outages because the same technical work can require a very different plan and price in an occupied facility.

State the place and anticipated period of performance.

Define normal and restricted work hours, blackout dates, and occupied-area limitations.

Identify notice periods for outages, access, noisy work, alarms, or system impairments.

Describe phasing, sequencing, temporary access, and coordination with other contractors or operations.

State milestone dates only when they are real requirements.

| Constraint | Requirement | Contractor must include |
| --- | --- | --- |
| Work hours |  |  |
| Access and security |  |  |
| Shutdown notice |  |  |
| Occupied-area controls |  |  |
| Phasing or sequencing |  |  |
| Required milestone |  |  |

# Step 8  Define deliverables and closeout

Name every document, physical item, demonstration, or record the contractor must provide. Include the required format, timing, review period, and acceptance procedure.

Why Scope-to-Go asks about this  The platform asks about submittals and closeout because the physical installation may be finished while warranties, manuals, training, test records, and asset information are still missing.

Product data, shop drawings, samples, permits, plans, and schedules.

Progress reports, inspection results, test reports, photographs, and deficiency records.

Training, demonstrations, keys, labels, spare parts, and asset information.

Warranty certificates, manuals, as-built records, final photographs, and turnover documents.

| Deliverable | Format or content | Due | Owner review or acceptance |
| --- | --- | --- | --- |

# Step 9  Write measurable acceptance standards

Replace vague words such as proper, adequate, clean, complete, or professional with observable conditions. Each important requirement should be capable of inspection, testing, measurement, or documented verification.

Why Scope-to-Go asks about this  The platform asks what success looks like because “complete” and “good workmanship” are difficult to enforce unless the finished condition can be observed, measured, tested, or documented.

Define the condition that must exist after the work is performed.

Identify the inspection, test, demonstration, document, or measurement used to verify it.

Name who reviews the result and what happens when it does not pass.

For recurring services, define the correction time and escalation path in the SLA.

| Requirement | Acceptance standard | Verification method | Correction requirement |
| --- | --- | --- | --- |
| Example: Finished floor | Uniform appearance; no visible gaps, raised edges, adhesive residue, damage, or trip hazards | Joint inspection and manufacturer-required tests | Correct before final acceptance |

## Vague language test

| Avoid | Replace with |
| --- | --- |
| Clean the area | Remove debris, dust, labels, residue, surplus material, and contractor equipment before reopening the area. |
| Install properly | Install according to the approved submittal and manufacturer instructions; verify operation through the stated test. |
| Match existing | Identify the exact finish, profile, dimension, color-selection process, or approved sample used for comparison. |
| Repair as needed | Define the observed condition that triggers repair and the required finished condition. |

# Step 10  Create a common pricing basis

Tell every contractor how to organize the estimate so material differences remain visible.

Why Scope-to-Go asks about this  The platform asks how contractors should price the work so totals, alternates, allowances, exclusions, and assumptions remain visible enough for an apples-to-apples comparison.

Request one base price tied to the complete SOW.

Separate owner-selected alternates from the base price.

Identify allowances only for genuinely unresolved quantities or selections.

Request unit prices for predictable variable work.

Require a written list of exclusions, assumptions, substitutions, qualifications, and proposed deviations.

| Price component | What the contractor must include |
| --- | --- |
| Base price | All labor, supervision, materials, equipment, delivery, protection, cleanup, and closeout required by the SOW unless specifically excluded. |
| Alternates | Separate price and schedule effect for each defined optional item. |
| Allowances | Included amount, permitted use, supporting documentation, and adjustment method. |
| Unit prices | Unit, inclusions, minimum charge, and conditions of use. |
| Disclosures | Every exclusion, assumption, substitution, qualification, and unresolved question. |

# Assemble and review the final document

Move the completed information into the recommended document order on page 1. Remove drafting prompts, reconcile conflicts, number the requirements, label attachments, and give the complete draft to someone who did not prepare it.

## Independent review questions

Could two qualified contractors reasonably interpret any important requirement differently?

Can each contractor identify the same work limits and responsibilities?

Are owner decisions complete, or clearly identified as pending?

Are verification items assigned before pricing rather than after award?

Does each major task lead to an observable result or required deliverable?

Can the facility manager explain how completion will be inspected and accepted?

Do the pricing instructions expose exclusions and assumptions instead of burying them?

Have required technical, legal, safety, code, and procurement reviews occurred?

## Final issue checklist

| Check | Ready |
| --- | --- |
| Purpose, scope boundaries, and existing conditions are clear. | ☐ |
| Tasks, materials, responsibilities, constraints, and protections are complete. | ☐ |
| Deliverables, schedule, acceptance, and correction requirements are stated. | ☐ |
| Pricing instructions create one common basis for estimates. | ☐ |
| All referenced exhibits are attached and consistently named. | ☐ |
| The approved version and issue date are recorded. | ☐ |

# How Scope-to-Go organizes the same process

The platform does not remove the facility manager from the process. It provides a guided sequence, keeps different kinds of information separated, and carries the approved scope into the later RFP, proposal-comparison, clarification, recommendation, and approval steps.

| Manual activity | How Scope-to-Go supports it |
| --- | --- |
| Interview stakeholders and gather records | Focused project questions, document inputs, asset information, photographs, and follow-up prompts. |
| Separate facts, decisions, and unknowns | Distinct handling for confirmed facts, facility-manager decisions, contractor verification, and specialist review. |
| Draft and reorganize the SOW | A structured, editable Prepared Scope of Work assembled from the reviewed answers. |
| Check for omissions and ambiguity | Technical review prompts that surface missing responsibilities, constraints, testing, closeout, and comparison risks. |
| Issue a consistent request | The approved scope stays connected to the RFP instructions and contractor outreach. |
| Reconcile different proposals | Proposal comparison and focused clarification tools preserve material differences before recommendation. |

# Sources used to structure this guide

Federal Acquisition Regulation 37.602, Performance Work Statement; U.S. Department of State Foreign Affairs Handbook 14 FAH-2 H-340, The Performance Work Statement; NASA Guidance for Writing Work Statements; and the U.S. Department of Energy project-management definition of Statement of Work. These sources informed the structure; this document is a practical educational guide, not a substitute for organizational procurement rules, legal review, engineering, code, or safety expertise.